COMPANION INTELLIGENCE

Xelulu — Gateway and Hardware Seam (the connection contract)

The single frozen contract where the hardware plugs into the software. Fill the boxes on the hardware side to match this, and the app, cloud and simulator work unchanged.

Document B  |  Version 2.0 (proto_ver 2)  |  8 October 2026  |  Owner: Founder/CTO (Claude) with Omar Saud (Founder)

Supersedes Version 1.0 (proto_ver 1), 7 Oct 2026. Change only by version bump, signed off by the software side (Claude/Omar) and the hardware side (intern). Based on the intern’s software spec and ElatoAI, with fixes applied.

Status: provisionally frozen on the founder/CTO’s rulings; the five proto_ver 2 changes (§11) are pending the intern’s confirmation. Build to this version now; patch only if the intern objects.

Timelines are intentionally excluded from this document. Costs and prices are planning estimates to be validated with supplier quotations.

Contents

# 1. What this document is

This is the seam: the one place where the robot (hardware) and the cloud/app (software) meet. Both sides build to this exact contract. Because the software is built and tested against a simulator that obeys this contract, the real robot "just works" the moment it obeys the same contract.

Rule: this contract changes only by a version bump (proto_ver), and only with sign-off from both sides. Everything else in the project can change freely; this cannot change quietly.

What changed in this version, in one line: OTA is now a command (the /ota topic is removed); the audio markers carry the envelope fields; events can be marked time-uncertain; the barge-in sequence is written down; and pairing moves to its own document. The seven touch zones, the MQTT+WebSocket split and Opus 16 kHz are unchanged. See §11 for the exact diff.

# 2. Transport and connection

| Property | Decision | Note |
| --- | --- | --- |
| Transport (control) | MQTT 5 over TLS | Status, events, commands, acks, OTA. Reliable pub/sub, auto-reconnect over flaky Wi-Fi, clean fleet messaging. |
| Transport (audio) | WebSocket (binary) over TLS | Opened only during a conversation turn; low-latency duplex Opus audio. |
| Security | TLS (wss://). Device authenticates on connect. | Prototype: a device token. Production: a per-device certificate or key kept in the ESP32-S3 secure storage. |
| Identity | device_id (permanent, one per robot), bound to one household_id at pairing | Issued by the cloud at provisioning. |
| Audio codec | Opus, 16 kHz mono, about 12–24 kbps, 20 ms frames | Both directions. |
| Heartbeat | Robot sends status every 15 seconds | Cloud marks the robot offline after 45 seconds of silence. |
| Versioning | proto_ver in every message (now 2) | If the robot’s and cloud’s versions differ, the robot stays on its last-known-good behaviour and reports the mismatch. |
| Reliability | Every command carries a cmd_id; every event carries an event_id | Commands are idempotent (safe to retry). Events can be de-duplicated after an offline period. |

Connection sequence:

robot powers on

-> connects to the MQTT broker (MQTT_BROKER_URL) over TLS; auth = device token / cert

-> publishes its first status (hello) on xelulu/{device_id}/status

-> subscribes to xelulu/{device_id}/cmd and xelulu/{device_id}/ack

-> cloud pushes the current config on xelulu/{device_id}/cmd

-> robot publishes status every 15 s and events as they happen

-> a conversation turn: robot opens the audio WebSocket (AUDIO_WSS_URL), streams

Opus up, cloud streams say-audio + a say command down, then the turn closes

-> OTA: cloud sends an `ota` command on /cmd (with cmd_id); robot acks and updates

-> on network loss: MQTT auto-reconnects; queued events (event_id + timestamp) are

published on reconnect; the cloud de-duplicates by event_id

Changed from v1: the robot no longer subscribes to a /ota topic — OTA now arrives as a command on /cmd (see §4 and §11).

## 2.1 MQTT topics

| Topic | Direction | Carries |
| --- | --- | --- |
| xelulu/{device_id}/status | device to cloud | heartbeat, battery, mode, fault (and the first one acts as hello) |
| xelulu/{device_id}/event | device to cloud | touch, wake, cry, motion, button, nfc, interrupt, error (each with event_id) |
| xelulu/{device_id}/cmd | cloud to device | say, animate, gesture, move, play, set_mode, led, volume, config, stop_audio, ota |
| xelulu/{device_id}/ack | device to cloud | command results by cmd_id |

Four topics (was five in v1). Audio is NOT on MQTT: it uses the separate audio WebSocket (AUDIO_WSS_URL), binary Opus, opened per turn. QoS: commands QoS 1 (idempotent via cmd_id); telemetry QoS 0 or 1.

# 3. Messages the robot sends up (device to cloud)

All JSON messages share an envelope: { "proto_ver": 2, "ts": <unix seconds>, "type": "<name>", ... }. Binary audio frames are sent as raw WebSocket binary during a conversation turn.

## 3.1 hello (first publish on xelulu/{device_id}/status)

{ "proto_ver": 2, "type": "hello", "device_id": "xlu_000123",

"fw_ver": "0.1.0", "auth": "<device token>", "proto": 2 }

## 3.2 status (every 15 seconds)

{ "proto_ver": 2, "ts": 1733400000, "type": "status",

"battery_pct": 72, "charging": false, "mode": "idle",

"wifi_rssi": -58, "fault": [], "active_child_id": "c_01",

"temp_c": 31 }

## 3.3 event (one envelope; event discriminates the kind) — every event has event_id

{ "proto_ver": 2, "ts": 1733400001, "type": "event",

"event_id": "ev_7f3a", "event": "touch",

"time_uncertain": false,

"data": { "zone": "belly", "gesture": "double_tap" } }

time_uncertain is new in v2 and optional. Set it true only when the robot generated the event while offline after a reboot, before its clock resynced; absent or false means the timestamp is trustworthy. The cloud orders such events as "time approximate" rather than trusting ts.

| event | data fields | Meaning |
| --- | --- | --- |
| touch | zone: head, left_ear, right_ear, belly, back, left_hand, right_hand; gesture: tap, double_tap, hold, pat | A child touched Xelulu |
| wake | source: hey_lulu or touch; dir_deg: direction of the voice if known | Wake word or touch-to-talk |
| cry | confidence: 0.0–1.0; duration_s | Crying detected. No audio is attached, ever. |
| motion | event: pickup, shake, upside_down, drop, tilt; value | Motion sensed |
| button | button: belly_talk; action: press, release | A button |
| nfc | card_id: story_042 | A story or code card was tapped (if NFC fitted) |
| interrupt | (none) | The child spoke over Xelulu; stop talking at once (see §3.5) |
| error | code, detail | A fault |

## 3.4 audio (during a conversation turn)

Each JSON marker now carries proto_ver and ts, like every other message (new in v2). The binary Opus frames between them are unchanged.

audio_start (JSON): { "proto_ver": 2, "ts": 1733400001,

"type": "audio_start", "turn_id": "t_9f" }

then binary Opus frames (raw WebSocket binary)

audio_end   (JSON): { "proto_ver": 2, "ts": 1733400004,

"type": "audio_end", "turn_id": "t_9f" }

## 3.5 Interrupt (barge-in) — the sequence

New in v2 as documentation; no new message is added. When the child speaks over Xelulu:

- Device sends the interrupt event (§3.3).

- Cloud stops TTS generation, sends audio_end for the open turn, then a stop_audio command (§4).

- Device stops playback on stop_audio and returns an ack.

There is no separate WebSocket "cancel" frame; interrupt + stop_audio are the whole mechanism.

# 4. Messages the cloud sends down (cloud to device)

Shared envelope: { "proto_ver": 2, "cmd_id": "<id>", "action": "<name>", "data": {...} }. The robot replies with an ack (section 5).

| action | data | Meaning |
| --- | --- | --- |
| say | audio: streamed Opus down the WS; expression; lang; voice (xelulu or grandma); duration_hint_ms | Speak with a synced face |
| animate | expression: happy, love, sleepy, curious, sad, listening, …; eyes: blink, hearts, …; loop | Face only |
| gesture | name: wave, high_five, hug, clap, point_left, nod, shake, … | Arms and head |
| move | cmd: come, stop, turn; linear_mms; angular_mrad_s; max_dist_mm | Bounded movement |
| play | content_id; source: stream or cache | Play a song or story |
| set_mode | mode: idle, play, story, quiet, sleep | Change mode |
| led | target: ears, cheeks, belly; color; effect: solid, pulse, rainbow; brightness | Lights |
| volume | level: 0–100 | Set volume |
| config | a settings delta: volume, quiet_hours, movement_on, language, active_child_id | Push settings |
| stop_audio | (none) | Stop speaking immediately (used after an interrupt) |
| ota | manifest_url | A firmware or content update is available. Carries a cmd_id and is acked (new in v2 — was a topic). |

## 4.1 Example — say

{ "proto_ver": 2, "cmd_id": "9f2c", "action": "say",

"data": { "expression": "happy", "lang": "en-IN", "voice": "xelulu",

"duration_hint_ms": 2200 } }

## 4.2 Example — ota (new form)

{ "proto_ver": 2, "cmd_id": "a1b2", "action": "ota",

"data": { "manifest_url": "https://.../manifest.json" } }

The robot acks ota like any command (accepted → done, or a rejected/failed with a reason). The retry-safety and delivery confirmation that OTA needs come from the cmd_id + ack machinery — which is why the separate /ota topic was removed.

# 5. Acknowledgements (device to cloud)

{ "proto_ver": 2, "cmd_id": "9f2c", "type": "ack",

"status": "done", "reason": "" }

status is one of: accepted, done, rejected, failed. The hardware is the source of truth for whether a command physically happened. When the robot refuses a command (lifted, at an edge, charging, quiet hours), it replies rejected with a reason from this list: picked_up, at_edge, charging, quiet_hours, low_battery, unsafe, busy, unknown_action.

# 6. On-device safety that the cloud must not override

- No movement while lifted, at an edge (stair or table), charging, or in quiet hours.

- Motor speed and torque capped in firmware; emergency stop independent of any AI.

- Wake word and cry detection run on the device; raw audio is never sent for a cry event.

- No flashing between 3 and 30 Hz (seizure safety).

- The robot keeps working offline for cached play, reminders and comfort, and queues events to send later (marking them time_uncertain when its clock is unsure — §3.3).

# 7. Fill the boxes — the hardware hookup

These are the exact values the hardware side provides so the connection comes alive. The software reads them from the single .env panel (Document A, section 10); the intern sets the matching values in the firmware. Fill each box once.

## 7.1 Boxes the CLOUD/APP side fills (you, via the .env panel)

| Box | What to paste | Where it comes from |
| --- | --- | --- |
| MQTT_BROKER_URL + MQTT_USERNAME / MQTT_PASSWORD | the control-plane broker | our deployed EMQX (Phase 1) |
| AUDIO_WSS_URL | the wss:// address for the audio turn | our audio gateway (Phase 1) |
| SUPABASE_URL / SUPABASE_ANON_KEY / SUPABASE_SERVICE_KEY | project URL and keys | your Supabase project settings |
| DEVICE_PROVISION_SECRET | a secret used to mint device tokens | we generate it in Phase 1 |
| LLM_API_KEY / STT_API_KEY / TTS_API_KEY | AI provider keys | the provider dashboards |

## 7.2 Boxes the HARDWARE side fills (the intern, in firmware)

| Box | What the intern sets | Must match |
| --- | --- | --- |
| DEVICE_ID | the robot’s permanent id | issued by our cloud at provisioning |
| MQTT_BROKER_URL + credentials | the control broker | the cloud’s MQTT_BROKER_URL |
| AUDIO_WSS_URL | the audio WebSocket address | the cloud’s AUDIO_WSS_URL |
| MQTT client | an ESP32 MQTT library (PubSubClient or ESP-MQTT) | subscribes /cmd; publishes /status, /event, /ack (no /ota topic) |
| DEVICE_TOKEN (prototype) / device key (production) | the robot’s auth | issued by our cloud |
| audio format | Opus, 16 kHz mono, 20 ms frames | section 2 |
| event names and fields | touch zones, gestures, motion, time_uncertain, etc. | section 3 exactly |
| command handling | say, animate, gesture, move, led, ota, etc. | section 4 exactly |
| ack reasons | the refusal reasons | section 5 exactly |

## 7.3 The hardware-to-pin reference (lives with the intern)

This contract does not fix GPIO pins; that is the intern’s hardware document. The link between the two is only the event and command names above. The intern maps, for example, "belly touch electrode" to the touch / belly event, and the led / ears command to the ear LED driver. Keep that mapping in the hardware document; keep this contract about messages only. That separation is what lets either side change wiring without breaking the other.

# 8. The simulator (how the software works with no robot)

Kiro builds a device simulator that connects to the gateway and behaves exactly like this contract: it sends hello, heartbeats and events (touch, wake, cry, motion) from a small control panel, accepts every command and returns an ack, and plays a fake audio turn so latency can be measured. It has an offline toggle to test queue-and-resync (and can mark replayed events time_uncertain). The app and cloud are developed entirely against it. Integration (Phase 6) is simply pointing the gateway at the real robot instead of the simulator — no change above the seam.

# 9. What the intern must confirm

| # | Question | Default if no objection |
| --- | --- | --- |
| 1 | MQTT (control) + WebSocket (audio) for version 1? | Confirmed by founder: yes |
| 2 | Firmware starts from ElatoAI’s firmware-arduino? | Yes |
| 3 | Audio: Opus, 16 kHz mono, 20 ms frames, both directions? | Yes |
| 4 | Touch zones = head, two ears, belly, back, two hands (seven)? | Yes (matches the current design) |
| 5 | On-device wake word and cry detection, no raw audio sent for cry? | Yes |
| 6 | The robot enforces the section 6 safety refusals and replies rejected with a reason? | Yes |
| 7 | OTA is a /cmd action (with cmd_id + ack), not a /ota topic? Drop any /ota subscription. | Yes |
| 8 | audio_start / audio_end carry proto_ver and ts? | Yes |
| 9 | Barge-in needs no new socket message: interrupt → audio_end → stop_audio; firmware stops playback on stop_audio? | Yes |
| 10 | Events gain an optional time_uncertain flag for offline-reboot events? | Yes |
| 11 | Wi-Fi/Bluetooth pairing moves to its own spec (Document C)? Flag any firmware assumption about pairing. | Yes |

When the intern confirms or changes any of these, tell me, and I will re-issue this document and update the simulator and the app to match.

# 10. Pairing and provisioning (moved out)

Wi-Fi and Bluetooth pairing — including the hand-off of the pairing token, binding a device_id to a household_id, and first-time Wi-Fi setup — are not part of this steady-state gateway contract. They move to Document C (Pairing and Provisioning), to be drafted when the Phase 1 simulator path is settled. Nothing here is blocked by it: the simulator and the contract assume an already-provisioned device.

# 11. Change log

| Version | Date | Change |
| --- | --- | --- |
| proto_ver 1 | 7 Oct 2026 | First frozen contract. MQTT (control) + WebSocket (audio); seven touch zones; added event_id, interrupt/stop_audio, led/volume commands, rejected reasons, and on-device safety. |
| proto_ver 2 | 8 Oct 2026 | 1. OTA is a command, not a topic. The /ota topic is removed; ota is an action on /cmd carrying cmd_id and returning an ack. Four topics remain (status, event, cmd, ack). (§2.1, §4) |
|  |  | 2. Audio markers obey the envelope. audio_start and audio_end now carry proto_ver and ts. (§3.4) |
|  |  | 3. Barge-in documented. interrupt (event) → cloud stops TTS, sends audio_end, then stop_audio (command) → device stops on stop_audio. No new socket message. (§3.5) |
|  |  | 4. time_uncertain flag. Optional boolean on an event, set true when the robot restarted offline before its clock resynced; the cloud treats the timestamp as approximate. (§3.3) |
|  |  | 5. Pairing split out. Wi-Fi/Bluetooth pairing and the pairing-token hand-off move to Document C. (§10) |
|  |  | Unchanged and still frozen: seven touch zones; MQTT + WebSocket split; Opus 16 kHz both directions; the rejected-reason list; the on-device safety rules. |

Confidential